/*!

\page create_filling_page Filling Surface with Edges

To generate a \b Filling in the <b>Main Menu</b> select <b>New Entity - > Generation  - > Filling</b>

To create a curvilinear face from several edges you need to define the

following parameters:

\n <b>Input Compound</b> - the list of edges/wires used for creation

of the surface. To prepare for the filling each wire of the compound

is converted to an edge created on a BSpline curve built using curves

from all edges of the wire.

\n \b Minimum and <b>Maximum Degree</b> of equation of the resulting

BSpline or Besier curves describing the surface;

\n \b Tolerance for \b 2D and for \b 3D - minimum distance between the

created surface and the reference edge;

\n <b>Number of Iterations</b> - defines the maximum number of iterations. The

iterations are repeated until the required tolerance is reached. So, a

greater number of iterations allows producing a better surface.

\n <b>Method</b> - Kind of method to perform filling operation

<ol>

<li>Default - the standard behaviour.</li>

<li>Use edges orientation - the edges orientation is used: if an edge is

reversed, the curve from this edge is reversed before being used by the filling

algorithm.</li>

<li>Auto-correct edges orientation - curves orientation is changed to

minimize the sum of distances between ends points of edges.</li>

</ol>

\n <b>Approximation</b> - if checked, BSpline curves are generated in

the process of surface construction (using

GeomAPI_PointsToBSplineSurface functionality). By default the surface

is created using Besier curves. The usage of <b>Approximation</b>

slows the algorithm, but allows building the surface for complex cases.

\n The \b Result of the operation will be a GEOM_Object (face).

\n <b>TUI Command:</b> <em>geompy.MakeFilling(Edges, MinDegree, MaxDegree, Tol2D, Tol3D, NbIter)</em>

\n <b>Arguments:</b> Name + 1 List of edges + 7 Parameters

(Min. degree, Max. degree, Number of iterations, 2D tolerance, 3D

tolerance, Number of iterations, Method, Approximation).

\n <b>Advanced options</b> \ref preview_anchor "Preview"

\image html filling.png

<b>Example:</b>

\image html filling_compoundsn.png "Initial edges"

\image html fillingsn.png "Resulting surface"

Our <b>TUI Scripts</b> provide you with useful examples of creation of

\ref tui_creation_filling "Complex Geometric Objects".

*/