/*!

\page glue_faces_operation_page Glue Faces

\n To <b>Glue Faces</b> in the <b>Main Menu</b> select <b>Repair - > Glue Faces</b>.

\n This operation glues faces that are coincident with respect to the

given tolerance value.

\n <b>Arguments:</b> Name + Compound of shapes + Tolerance value.

\n <b>Advanced option:</b>

\ref restore_presentation_parameters_page "Set presentation parameters and sub-shapes from arguments".

\n The \b Result will be a \b GEOM_Object.

\image html glue1.png

\n <b>TUI Commands:</b>

\n <em>geompy.MakeGlueFaces(theShape,theTolerance,doKeepNonSolids)</em>,

\n where \em theShape is a compound of shapes to be glued, \em

theTolerance is a maximum distance between two faces, which can

be considered as coincident. The \em doKeepNonSolids flag allows to

throw away non-solids from the result, if false. The \b Result will

be a new \b GEOM_Object.

\n It is also possible to manually select the faces that will be

glued - select the shape, specify the tolerance and press \b Detect button.

\image html glue2.png

\n \b Geometry module detects the faces where gluing can be

performed and displays a notification.

\image html glue3.png

\n The faces that can be glued are colored in red. It is

possible to select the faces for gluing in the 3D viewer.

The selected faces will be marked in white.

\n When the faces are glued their edges are glued as well. By default, other

edges are not glued. To force gluing of all edges, check <b>Glue all coincident edges</b>

checkbox.

\n <b>TUI Commands:</b>

\n <em>geompy.GetGlueFaces(theShape,theTolerance)</em>,

\n where \em theShape is a compound of shapes to be glued, \em

theTolerance is a maximum distance between two faces, which can

be considered as coincident. The \b Result will be a list of \b

GEOM_Objects, containing one sub-shape per each detected set of

coincident sub-shapes.

\n <em>geompy.MakeGlueFacesByList(theShape,theTolerance,theFaces,doKeepNonSolids,doGlueAllEdges)</em>,

\n where \em theShape is a compound of shapes to be glued, \em

theTolerance is a maximum distance between two faces, which can

be considered as coincident, \em theFaces is a list of

sub-shapes to be glued. The \em doKeepNonSolids flag allows to throw

away non-solids from the result, if false. The \em doGlueAllEdges

allows to glue all edges, not only owned by glued faces. The \b

Result will be a new \b GEOM_Object.

\n <b>Example:</b>

\image html glue_faces1.png

<center><em>Object that needs gluing</em></center>

\image html glue_faces2.png

<center><em>Glued object</em></center>

\image html glue_faces3.png

<center><em>Manual selection of faces for gluing</em></center>

Our <b>TUI Scripts</b> provide you with useful examples of the use of

<b>Repairing Operations</b> \ref tui_glue_faces "Glue Faces".

*/