/*!

\page create_extrusion_alongpath_page Pipe Construction

To generate a \b Pipe in the <b>Main Menu</b> select <b>New Entity - > Generation  - > Extrusion along a path</b>

\n Firstly, to create an extruded \b Pipe shape, you can define the <b>Base

Object</b> (vertex, edge, planar wire, face or shell), which will be extruded

and the <b>Path Object</b> (edge or wire) along which the <b>Base

Object</b> will be extruded.<br>

It is possible to select in GUI several Base Objects to make several pipes (using Shift button).

\n In this mode the default pipe generation algorithm is used. The pipe

is created so that the <b>angle between the normal vector</b> to

the base shape <b>and the tangent to the path</b> wire remains constant at any point of the

given path.

\n <b>Select unpublished edges</b> checkbox - if checked, allows to select an edge of any other shape as the path object.

\n The \b Result of the operation will be a GEOM_Object (edge, face, shell,

solid or compsolid).

\n <b>TUI Command:</b> <em>geompy.MakePipe(baseShape, pathShape)</em>

\n <b>Arguments:</b> Name + 1 shape (vertex, edge, planar wire, face or

shell) serving as base object + 1 shape (edge or wire) for

definition of the path.

\n <b>Advanced options</b> \ref preview_anchor "Preview"

\image html pipe.png

<b>Example:</b>

\image html pipe_wire_edgesn.png "Extrusion of a wire"

\n Secondly, you can define the <b>Base

Object</b> (edge, planar wire or face), which will be extruded,

the <b>Path Object</b> (edge or wire) along which the <b>Base

Object</b> will be extruded and the <b>BiNormal Vector</b> (edge or

wire).

\n In contrast to the default mode, in this mode the pipe is generated

to preserve the constant <b>angle between the normal vector</b> to the base

shape <b>and the BiNormal vector</b> given as the third argument.

\n <b>Select unpublished edges</b> checkbox - if checked, allows to select an edge of any other shape as the path object.

\n The \b Result of the operation will be a GEOM_Object (edge, face, shell,

solid or compsolid).

\n <b>TUI Command:</b> <em>geompy.MakePipeBiNormalAlongVector(baseShape, pathShape, binormalShape)</em>

\n <b>Arguments:</b> Name + 1 shape (edge, planar wire or face)

serving as base object + 1 shape (edge or wire) for

definition of the path + 1 shape (edge or wire) to set a fixed

BiNormal direction to perform the extrusion.

\image html pipe2.png

<b>Example:</b>

\image html pipebinormalsn.png "Extrusion of a face with BiNormal parameter"

Our <b>TUI Scripts</b> provide you with useful examples of creation of

\ref tui_creation_pipe "Complex Geometric Objects".

*/