/*!

\page extruded_cut_operation_page Extruded cut

The <b>Extruded cut</b> operation allows you to easily remove material from a \b solid by extruding a profile along the normal to its plane.

\image html extruded_cut_example.png

To produce the <b>extruded cut</b>:

<ol>

<li> Draw the \b profile of the extrusion. It can be:

<ul>

<li> a <a href="create_sketcher_page.html">\b Sketch </a> drawn on a planar face of the object you want to cut.</li>

<li> any closed edge or wire of the desired shape (circle, ellipse ...).</li>

</ul>

</li>

<li> In the <b>Main Menu</b> select <b>Operations - > Extruded Cut</b> or click on

\image html extruded_cut.png

</li>

<li> Fill in the dialog box fields:

\image html extruded_cut_dlg.png

</li>

</ol>

The input arguments are:

- The <b>initial shape</b> you want to cut. It must be a solid or a compound made of a single solid.

- The \b profile. It must be a \b closed edge or wire and it must be \b planar.

- The \b height of extrusion. It's calculated from the <b>base profile</b> along the normal to its plane.

And optionnaly:

- A <b>draft angle</b> in degrees. Activated by clicking on \image html draft.png

The direction can be reversed by pushing the <b> Change direction </b> button: \image html change_direction.png

This functionality is available via the textual interface with the command:

\n <b>TUI Command:</b> <em>geompy.MakeExtrudedCut(InitShape, Profile, Height,Angle),</em>

*/